Image Acquisition

Go to Acquire

1

1

2

Choose a single setting from the list (i.e. FITC/TRITC) and continue with step 6.

Steps 2 to 5 are automatically set.

7

6

7

3

5

If no appropriate single setting is available, proceed manually with steps 2 to 5 :

Start Live scan*.

Click QLUT to switch to Glow.

Change Z Position by turning

the respective wheel on the control panel until you get the brightest signal (focal plane).

8

2

4

3

4

Activate lasers (ON).

Set excitation laser line(s). Activate PMT/HyD (ON), defne Gain and Offset.

5 Position PMT slider at

maximum emission.

8

6

8

Optimize image settings manually by adjusting steps

3 to 5 , until the image contains a few blue (saturated) pixels.

Go back to original LUT 7

by clicking twice.

Capture Image*.

Save Project (see below, Z-Series Acquisition).

Z-Series Acquisition

1

2

3

4

Open Z-Stack Dialog. Set Focal Plane.

Set Begin- and End-Position. Click System Optimized to defne the number of slices.

Click Start to acquire the z-series*.

7

8

6

7

6

Click Max. to generate a maximum projection.

To save the experiment go to tab Project and

8 click on Save.

5

1

3

2

4

5

System Shut Down

Follow exactly the sequence described to shut down the system:

1. Deactivate all lasers in the laser confguration window (see Laser Start Up section), wait approx. 5 min. until ventilation of Argon laser has shut-off. 2.

3. Shut down PC.

4. Shut down CSU or FSU, respectively, in reverse order (see System Start Up section).

* CAUTION!

Laser light in objective plane from now on. User manual must be strictly observed.

System Start Up

1

2

3

1	2

3

4

Compact Supply Unit (CSU)

Flexible Supply Unit (FSU)

Turn on the microscope control box.

For system confguration with CSU: Turn on the PC and await the booting process. Then turn on the power supply for the scanner 1 and the lasers 2 by pushing the green buttons and turn the

laser key 3 , to open the laser shutter.

For system confguration with FSU: Turn on the power supply for the PC 1 , the scanner 2 and the lasers 3

by pushing the green buttons and turn the laser key 4 , to open the

laser shutter.

Laser Start Up

1

2

Go to

Configu-ration.

- Activate lasers (ON)

- Use the slider to set the laser power of the Argon laser (458 nm–514 nm) to 20–30 %.

# QUICK START – LEICA TCS SP8

Software Start Up

Start LAS X

by double-clicking the LAS X icon.

Make sure there is the correct selection for Confguration and Microscope.

Turn on Apply Customized User Settings to choose User Settings saved in a preceding session. Turn off Apply Customized User Settings to start LAS X with the default settings. Click OK and follow the instructions (i.e. Initialize Stage).